Introduction — working draft

Paper 1, DATE. Target ~0.75 page. P1 is drafted; P2–P5 give the points plus suggested text to take from and rephrase. Numbers carry their source so rewording cannot break a fact.

P1 — The stage (drafted)

Deep Neural Networks (DNNs) have become the dominant workload class across computing. They underpin the vast majority of AI-based applications, and DNN inference has become infrastructure, running continuously from edge devices to warehouse-scale data centers [TPU]. The latency and energy budget of DNN accelerators is therefore a first-order design metric, and a primary axis of competition among chip designers and manufacturers. Weight movement is what drives this budget in conventional accelerators [Horowitz, Sze]. In-Memory Computing (IMC) eliminates weight traffic by keeping weights in place and performing Multiply-Accumulate (MAC) operations locally. This alleviates the weight-movement bottleneck, improving performance while reducing the communication energy budget and launching a new generation of IMC DNN accelerators [Sebastian, ISAAC, PRIME].

However, a single array holds nowhere near enough weights for a full DNN, making many-tile architectures inevitable. For example, one ResNet-50 block requires 736 crossbars (CBs) and a full network requires over ten thousand. Because ADC peripherals have a large area and power overhead, they are shared across the arrays of a tile. This limits the number of CBs that fit in a tile, resulting in the computation being split among hundreds or even thousands of tiles [ISAAC].

The consequence is that IMC does not eliminate communication; it relocates it. What remains mobile is activation scatter traffic and, dominantly, partial-sum reduction: at the default packing, roughly 82% of a block’s bytes converge on a few accumulator tiles via the on-chip network. The bottleneck that governs both performance and energy therefore becomes communication, and the design focus shifts from computation-centric to communication-centric.

Verified: 736 CBs = conv1 4×16 + conv2 18×16 + conv3 2×64 + shortcut 4×64. 82% is the reduction share at (8,1,8) — it is 45.6% at (8,2,4), hence “at the default packing”. 20 accumulator tiles of 92 active. ISAAC’s per-tile crossbar figure is still on the open-items list — confirm before it anchors §3.

P2 — Hook A: the wrong objective

•  How crossbars are grouped onto tiles — the packing orientation (r,s) at fixed per-tile density c — decides how much reduction stays on-tile and how much reaches the network.

•  Practice selects the packing on pre-NoC objectives: utilisation, tile count, communication volume.

•  The packing fixes a placement-invariant port floor — a per-node injection/ejection rate no later mapping or routing decision can lower.

•  Result: minimum-volume and minimum-floor orientations differ in 5 of the 8 (workload, density) cells where a choice exists.

Suggested: “How the crossbars are grouped onto tiles — the packing orientation (r,s) at a fixed per-tile density c — decides how much of that reduction is internalised on-tile and how much reaches the network. Practice selects this packing on pre-NoC objectives: utilisation, tile count, communication volume. We show these objectives pick the wrong point. The packing fixes a placement-invariant port floor that no later mapping or routing decision can lower, and the minimum-volume orientation is not the minimum-floor orientation in five of the eight (workload, density) cells where a choice exists: at c = 16, ResNet-50’s minimum-volume packing carries a 2.78× higher floor than one costing only 8% more traffic.”

Alternative headline if VGG reads better: c = 32, 2.57× floor for 7% traffic. DeiT-S is the honest counterweight — 1.8–3.2× floor reduction but at 1.6× the traffic.

P3 — Hook B: the two regimes

•  The floor splits the design space in two, and which side a design sits on is decided at packing, before any mapping effort.

•  Below it: placement moves delay only in a band around the congestion knee — 1.34× at light load, 13.6× at the knee, 1.28× in saturation.

•  Above it: placement takes control — 4.6–31.8× delay spreads across placements, all three workloads.

•  Optional clause: hop count sets where the delay curve starts; the binding load sets where it bends.

Suggested: “That floor splits the design space into two regimes, and which one a design occupies is decided at packing, before any mapping effort is spent. Below the floor, placement moves delay only within a band around the congestion knee. Above it, placement takes control outright, with delay spreads of 4.6–31.8× across placements of a single packing on all three workloads. The same optimisation is near-worthless in one regime and decisive in the other.”

P4 — Contributions (four bullets)

•  Layer attribution: the port floor is the packing’s quantity (placement-invariant, 5.8× span across orientations); peak link load is the placement’s (7.5× within one packing). Their maximum is where the layers meet.

•  A regime map: below the floor the orientation sets achievable delay (1.56–1.80× capacity at fixed density) and minimum-hop mapping captures nearly all of the rest; above it placement takes control. Peak link load predicts capacity (negative in 5 of 5 populations); escape-room metrics do not.

•  Two faces of adaptive selection: above the floor it is an unconditional throughput lever (1.15× delay, 1.36× p99, 65/71 wins); at the knee it is a robustness lever — it compresses the delay spread across placements (8 of 8 populations, up to 7.5×) by rescuing the worst ones, returning the mapping freedom congestion took away.

•  A negative result with a design consequence: below the floor the policy choice is placement-specific and offline-unpredictable — a per-placement oracle beats always-adaptive by 4.2% on delay and 11.6% on p99, and no offline metric calls it. The residual is real and needs an online signal.

Open: does the design rule (orientation on PF → min-CC map → then min PL → adaptive everywhere for tail) close this paragraph, or wait for Results? Claim numbers stay out of the paper.

P5 — Scope and organisation

Suggested: “All results are cycle-accurate simulation on one 6×6×3 substrate over ResNet-50, VGG-16 and DeiT-S blocks (approximately 20,600 runs, no failures). Thermal effects appear only as motivation for constrained placements, and the learned selection policy the final result motivates is left to follow-on work.”

Then one organisation sentence.

Numbers used, with provenance

•  736 crossbars per ResNet-50 block; over 10,000 for the full network (25.6 M params / 2,048 weights per 128×128 array).

•  92 tiles at c = 8 on a 108-node mesh; ~1,560 tiles for the full network.

•  82% reduction share at (8,1,8); 45.6% at (8,2,4) — packing-specific.

•  20 accumulator tiles of 92 active; two hottest absorb 32–38% of traffic.

•  Port floor spans 5.8× over ResNet’s 11 feasible points; peak link load spans 7.5× within one packing.

•  5 of 8 cells where minimum-volume and minimum-floor disagree; 2.78× floor for 8% traffic (ResNet c=16); 2.57× for 7% (VGG c=32).

•  Below-floor placement spread: 1.34× / 13.6× / 1.28× across load.

•  Above-floor placement spread: 4.6–31.8×, three workloads.

•  Adaptive selection above floor: 1.153× delay, 1.364× p99, 65/71.

•  Spread compression: 8 of 8 qualifying populations, best 7.50×.

•  Oracle vs always-adaptive below floor: 1.042× delay, 1.116× p99.

P1 — the stage.
Deep-neural-network inference has become infrastructure: it runs continuously from edge devices to datacenters, and its energy and latency budget — not raw arithmetic — is what accelerator design answers to. In conventional accelerators that budget is dominated by data movement, weight fetch above all [cite]. In-memory computing (IMC) attacks this directly: weights stay resident in crossbar arrays and multiply-accumulate happens in place, eliminating weight traffic entirely and driving the current generation of IMC DNN accelerators [ISAAC/PRIME/survey]. But a real network needs hundreds of crossbars spread across many tiles — 736 for one ResNet-50 block — so IMC does not remove communication; it relocates it. What remains mobile is activations and, dominantly, partial-sum reduction: roughly 82% of the block's bytes converge on a handful of accumulator tiles across the on-chip network. The accelerator's bottleneck becomes a communication problem, and this paper studies it on a 3D NoC.

P2 — hook-a.
How the crossbars are grouped onto tiles — the packing orientation $(r,s)$ at a fixed per-tile density $c$ — decides how much of that reduction is internalised on-tile and how much reaches the network. Practice selects this packing on pre-NoC objectives: utilisation, tile count, communication volume. We show these objectives pick the wrong point. The packing fixes a placement-invariant port floor — a per-node injection/ejection rate that no later mapping or routing decision can lower — and the minimum-volume orientation is not the minimum-floor orientation in five of the eight (workload, density) cells where a choice exists: at $c{=}16$, ResNet-50's minimum-volume packing carries a 2.78× higher floor than one costing only 8% more traffic.

P3 — hook-b.
That floor splits the design space into two regimes, and which one a design occupies is decided at packing, before any mapping effort is spent. Below the floor, placement moves delay only in a band around the congestion knee — a 3.07× spread in peak link load moves mean delay 1.15× at light load, yet 13.6× at the knee. Above it, placement takes control outright: 4.6–31.8× delay spreads across placements of a single packing, on all three workloads. The same optimisation is near-worthless in one regime and decisive in the other.

P4 — contributions. (the four bullets, as agreed — layer attribution / regime map / two faces of adaptivity / the offline-unpredictable residual — plus the design-rule sentence if q3 resolves to "intro")

P5 — scope.
All results are cycle-accurate simulation on one 6×6×3 substrate over ResNet-50, VGG-16 and DeiT-S blocks (~20,600 runs); thermal appears only as motivation for constrained placements, and the learned selection policy the final result motivates is deferred to follow-on work. (+ organisation sentence)